# Packet Tracer - Configure Router-on-a-Stick Inter-VLAN Routing

# Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0.10 | 172.17.10.1 | 255.255.255.0 | N/A |
| R1 | G0/0.30 | 172.17.30.1 | 255.255.255.0 | N/A |
| PC1 | NIC | 172.17.10.10 | 255.255.255.0 | 172.17.10.1 |
| PC2 | NIC | 172.17.30.10 | 255.255.255.0 | 172.17.30.1 |

# Objectives

Part 1: Add VLANs to a Switch

Part 2: Configure Subinterfaces

Part 3: Test Connectivity with Inter-VLAN Routing

# Scenario

In this activity, you will configure VLANs and inter-VLAN routing. You will then enable trunk interfaces and verify connectivity between VLANs.

# Instructions

## Add VLANs to a Switch

### Create VLANs on S1.

Create VLAN 10 and VLAN 30 on S1.

Open configuration window

### Assign VLANs to ports.

Configure interfaces F0/6 and F0/11 as access ports and assign VLANs.

Assign the port connected to PC1 to VLAN 10.

Assign the port connected to PC3 to VLAN 30.

Issue the show vlan brief command to verify VLAN configuration.

S1# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

Fa0/5, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/12, Fa0/13, Fa0/14

Fa0/15, Fa0/16, Fa0/17, Fa0/18

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gig0/1, Gig0/2

10   VLAN0010                         active    Fa0/11

30   VLAN0030                         active    Fa0/6

1002 fddi-default                     active

1003 token-ring-default               active

1004 fddinet-default                  active

1005 trnet-default                    active

Close configuration window

### Test connectivity between PC1 and PC3.

From PC1, ping PC3.

#### Question:

Were the pings successful? Why did you get this result?

Type your answers here.

## Configure Subinterfaces

### Configure subinterfaces on R1 using the 802.1Q encapsulation.

Open configuration window

Create the subinterface G0/0.10.

Set the encapsulation type to 802.1Q and assign VLAN 10 to the subinterface.

Refer to the Address Table and assign the correct IP address to the subinterface.

R1(config)# int g0/0.10

R1(config-subif)# encapsulation dot1Q 10

R1(config-subif)# ip address 172.17.10.1 255.255.255.0

Repeat for the G0/0.30 subinterface.

### Verify Configuration.

Use the show ip interface brief command to verify subinterface configuration. Both subinterfaces are down. Subinterfaces are virtual interfaces that are associated with a physical interface. Therefore, in order to enable subinterfaces, you must enable the physical interface that they are associated with.

Enable the G0/0 interface. Verify that the subinterfaces are now active.

Close configuration window

## Test Connectivity with Inter-VLAN Routing

### Ping between PC1 and PC3.

#### Question:

From PC1, ping PC3. The pings should still fail. Explain.

Type your answers here.

### Enable trunking.

Open configuration window

On S1, issue the show vlan command.

#### Question:

What VLAN is G0/1 assigned to?

Type your answers here.

Because the router was configured with multiple subinterfaces assigned to different VLANs, the switch port connecting to the router must be configured as a trunk. Enable trunking on interface G0/1.

#### Question:

How can you determine that the interface is a trunk port using the show vlan command?

Type your answers here.

Issue the show interface trunk command to verify that the interface is configured as a trunk.

Close configuration window

### Test Connectivity

If the configurations are correct, PC1 and PC3 should be able to ping their default gateways and each other.

#### Question:

What addresses do PC1 and PC3 use as their default gateway addresses?

Type your answers here.

end of document