# Packet Tracer - Troubleshoot Inter-VLAN Routing

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | VLAN |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1.10 | 172.17.10.1 | 255.255.255.0 | N/A | VLAN 10 |
| R1 | G0/1.30 | 172.17.30.1 | 255.255.255.0 | N/A | VLAN 30 |
| PC1 | NIC | 172.17.10.10 | 255.255.255.0 | 172.17.10.1 | VLAN 10 |
| PC3 | NIC | 172.17.30.10 | 255.255.255.0 | 172.17.30.1 | VLAN 30 |

# Objectives

Part 1: Locate Network Problems

Part 2: Implement the Solution

Part 3: Verify Network Connectivity

# Scenario

In this activity, you will troubleshoot connectivity problems caused by improper configurations related to VLANs and inter-VLAN routing.

# Instructions

## Locate the Network Problems

Examine the network and locate the source of any connectivity issues.

Open configuration window

Commands you may find useful include:

R1# show ip interface brief

R1# show interface g0/1.10

R1# show interface g0/1.30

S1# show interface trunk

Test connectivity and use the necessary show commands to verify configurations.

Verify that all configured settings match the requirements shown in the Addressing Table.

List all of the problems and possible solutions in the Documentation Table.

Documentation Table

| Problems | Solutions |
| --- | --- |
| Blank | Blank |
| Blank | Blank |
| Blank | Blank |
| Blank | Blank |
| Blank | Blank |
| Blank | blank |

## Implement the Solutions

Implement your recommended solutions.

Close configuration window

## Verify Network Connectivity

Verify the PCs can ping each other and R1. If not, continue to troubleshoot until the pings are successful.

End of document