# Packet Tracer - Inter-VLAN Routing Challenge

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 172.17.25.2 | 255.255.255.252 | N/A |
| R1 | G0/1.10 | 172.17.10.1 | 255.255.255.0 | N/A |
| R1 | G0/1.20 | 172.17.20.1 | 255.255.255.0 | N/A |
| R1 | G0/1.30 | 172.17.30.1 | 255.255.255.0 | N/A |
| R1 | G0/1.88 | 172.17.88.1 | 255.255.255.0 | N/A |
| R1 | G0/1.99 | 172.17.99.1 | 255.255.255.0 | N/A |
| S1 | VLAN 99 | 172.17.99.10 | 255.255.255.0 | 172.17.99.1 |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 | 172.17.10.1 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 | 172.17.20.1 |
| PC3 | NIC | 172.17.30.23 | 255.255.255.0 | 172.17.30.1 |
| Server | NIC | 172.17.50.254 | 255.255.255.0 | 172.17.50.1 |

# VLAN and Port Assignments Table

| VLAN | Name | Interface |
| --- | --- | --- |
| 10 | Faculty/Staff | F0/11-17 |
| 20 | Students | F0/18-24 |
| 30 | Guest(Default) | F0/6-10 |
| 88 | Native | G0/1 |
| 99 | Management | VLAN 99 |

# Scenario

In this activity, you will demonstrate and reinforce your ability to implement inter-VLAN routing, including configuring IP addresses, VLANs, trunking, and subinterfaces.

# Instructions

Configure the devices to meet the following requirements.

Assign IP addressing to R1 and S1 based on the Addressing Table.

Configure the default gateway on S1.

Create, name, and assign VLANs on S1 based on the VLAN and Port Assignments Table. Ports should be in access mode. Your VLAN names should match the names in the table exactly.

Configure G0/1 of S1 as a static trunk and assign the native VLAN.

All ports that are not assigned to a VLAN should be disabled.

Configure inter-VLAN routing on R1 based on the Addressing Table.

Verify connectivity. R1, S1, and all PCs should be able to ping each other and the server.

End of document